Introduction:

Separated internal flows are commonly encountered in engineering systems such as diffusers, combustors, ventilation ducts, pipe expansions, and aerodynamic components. When a flow passes over a sudden expansion or backward-facing step, the adverse pressure gradient causes the boundary layer to separate from the wall, forming a recirculation region downstream of the step. Farther downstream, the separated shear layer develops, loses momentum, and eventually reattaches to the wall. The location of this reattachment point is an important flow quantity because it indicates the size of the separation bubble and strongly affects pressure loss, momentum recovery, and wall shear stress distribution [1].

The present study investigates turbulent separated flow based on the experiment conducted by Pitz and Daily (1981), which represents a two-dimensional backward-facing step internal flow [2]. The numerical model is developed using the standard OpenFOAM pitzDaily tutorial case. This case is useful for studying flow separation, recirculation, and downstream recovery because it contains a sudden geometric expansion that produces a clearly defined separated shear layer. The flow is solved in OpenFOAM using the steady-state incompressible solver simpleFoam. A Reynolds-averaged Navier–Stokes turbulence model is used to approximate the turbulent flow field.

The main objective of this project is to evaluate the effect of inlet velocity on the separated flow structure in the pitzDaily case. Four inlet velocities are considered: 5 m/s, 10 m/s, 15 m/s, and 20 m/s. This velocity range allows the influence of increasing inertial effects to be studied systematically while keeping the geometry, turbulence model, and boundary-condition setup unchanged.

Before carrying out the inlet-velocity parametric study, a mesh independence study is performed using coarse, medium, and fine meshes. This step ensures that the numerical results are not strongly dependent on grid resolution. The mesh-independent solution is then used for parametric study to compare the effect of inlet velocity on the flow field. The final results are analyzed using velocity contours, pressure contours, streamline or glyph visualizations, and reattachment length measurements.

To validate the numerical simulation, the numerical model is compared with an experimental reference case with an inlet velocity of 9.1 m/s. The reattachment length is selected as a key comparison parameter because it provides a quantitative measure of the separated flow region and allows direct comparison between the numerical and experimental results.

Numerical Setup

Mesh Sensitivity Study

Parametric Study

Discussion

Reference:

[1] eaton1981

[2] pitz1981